# High-Level Overview of Vision-Enhanced 2-Wheel Robot Software Development Approach

This is a recap of the structured, iterative approach we discussed for your vision-enhanced 2-wheel robot (built around Teensy 4.1 for low-level motor control + Raspberry Pi 5 with OAK-D Lite for high-level vision/spatial AI). The goal is a modular, maintainable codebase that supports real-time object detection/avoidance, odometry/IMU fusion, navigation in your lab space, and eventual AprilTag-based charging station homing/docking — starting with tethered testing (~1m range).

- Overall Architecture (Hybrid Teensy + Pi5)

Teensy 4.1 (Low-Level / Real-Time): Handles motor control, Hall effect encoders (odometry), MPU6050 IMU (fusion for heading/pose), safety stops, and basic sensor loops. Uses your custom FlasherX for OTA updates. Runs at high frequency for tight control loops.

Raspberry Pi 5 (High-Level / Vision): Runs DepthAI (with OAK-D Lite's Myriad X VPU) for spatial object detection, depth mapping, occupancy grid generation, and path planning. Communicates with Teensy via serial (UART) or other reliable link for commands and status.

Communication: Bidirectional — Pi sends velocity/heading commands or waypoints; Teensy reports odometry/IMU data, encoder ticks, and sensor status. Start simple (text/JSON packets) and evolve to a more robust protocol if needed.

Shared Code: Use a Teensy_Robot_Base (or similar) repo for common utilities (OTA/FlasherX, IMU fusion, motor PID, comms protocol). Individual projects (e.g., my_vision_robot) pull in the base as a submodule or library.

- Development Workflow & Tooling

IDE: Stick with Visual Studio + Visual Micro for Teensy side (your preference and familiarity). Use it for C++ on Teensy. For Pi side, Python (DepthAI/OpenCV if needed) can be developed in VS as well or VS Code.

Project Structure (custom, future-proof):

C:\Projects\Teensy_Robot_Base\ — shared code, .gitignore tuned for Arduino/Teensy.

C:\Projects\my_vision_robot\ (or TwoWheelRobot) — project-specific code, references base.

Avoid default Arduino folders; use your preferred paths with clear naming.

Version Control: Git (command-line now). Modular repos with submodules for base code. Commit early/often, especially after successful tethered tests.

Build/Test Cycle:

Develop/test Teensy low-level first (motors, encoders, IMU, comms).

Add Pi vision pipeline.

Integrate comms + closed-loop control.

Iterative tethered runs in lab → untethered.

- Software Layers / Modules (Incremental)

Low-Level (Teensy):

Motor control + PID.

Encoder odometry.

IMU fusion (e.g., Madgwick or complementary filter).

Comms parser (receive commands, send telemetry).

Safety (watchdog, tether limit detection if applicable).

Vision & Perception (Pi + DepthAI):

Camera pipeline: RGB + depth + neural network (object detection — you mentioned no prior OpenCV/YOLO, so start with DepthAI's built-in models or easy NN blobs).

Spatial mapping: Convert detections to world coordinates using depth.

Occupancy grid or simple costmap for obstacles.

Navigation & Planning (Pi):

Basic behaviors: "go toward exit" while avoiding obstacles.

Path planning (A*, DWA, or even simple potential fields initially).

AprilTag detection for docking station (later phase).

High-Level Control:

State machine on Pi (e.g., Explore, Avoid, Dock, Idle).

Send velocity commands to Teensy.

- Testing & Iteration Plan

Phase 1 (Tethered): Camera assesses space, plans simple path toward room exit, Teensy executes open-loop or basic closed-loop motion. Stop short of tether limit.

Phase 2: Add obstacle avoidance using depth data.

Phase 3: IMU + encoder fusion for better pose estimation.

Phase 4: AprilTag homing/docking.

Debugging: Serial logging, visual overlays on Pi (e.g., display detections), telemetry to host PC.

- Key Considerations

Real-Time vs. Compute: Keep heavy vision/AI on Pi; Teensy stays fast and predictable.

Power & Heat: Monitor during extended tethered tests.

Modularity: Abstract interfaces (e.g., MotorController, VisionProcessor, Navigator) so you can swap components easily.

Libraries: DepthAI for vision, Teensy-specific libs for IMU/encoders, something lightweight for comms/fusion.